Joystick Module Document :

Author : Sanjyot Kadu

Date : 17-02-2026

Version No : joystick_Module_doc_001

# Change History:

| Date | Version No | Change History |
| --- | --- | --- |
| 17-02-2026 | joystick_Module_doc_001 | First Revision |

# Subject:

This module is part of Middleware and provides information about joystick behaviour and all other necessary information about it.

# Configuration Table:

| Sr No | Signal Name | Value | Type |  |
| --- | --- | --- | --- | --- |
| 1 | cf_ADC_positive_range_min | 10 |  |  |
| 2 | cf_ADC_positive_range_max | 2030 |  |  |
| 3 | cf_ADC_Neutral_range_min | 2031 |  |  |
| 4 | cf_ADC_neutral_range_max | 2060 |  |  |
| 5 | cf_ADC_negative_range_min | 2061 |  |  |
| 6 | cf_ADC_negative_range_max | 4000 |  |  |
| 7 | cf_Min_ADC_Value | 0 |  |  |
| 8 | cf_Max_ADC_Value | 4095 |  |  |
| 9 | cf_Min_Joy_Value | 0 |  |  |
| 10 | cf_Max_Joy_Value | 100 |  |  |

# Requirements:

## When 12 bit or 8 Bit ADC value is received , it shall be converted into Joystick Value and Joystick Direction.

Function can be

### retVal getJoyInfoFromADC(ADC_Signal_cong, ADC_Value, JoystickInfo)

- function name : getJoyInfoFromADC()

- Inputs :

- ADC_Siganl_Conf : This can be 8 ,12 or 16 (as per configured ADC)

- ADC_Value : This is value received from ADC , so for 12 bit ADC the range will be from  0 to 2^12(0 to 4095)

- OutPut:

joyStickInfo_t (structure)

joystickDir  (structure memeber) – Can be value

if from   0 to 2030 then its (positive direction (forward or left))

if from   2030 to 30 then its (neutral direction)

if from   30 to 4095 then its (negative direction (back or right)))

configuration:

values of cf_positive_range_min , cf_positive_range_max

values of cf_Neutral_range_min and cf_neutral_range_max

values of cf_negative_range_min and cf_negative_range_max

shall be taken from configuration file.

- JoystickVal(structure memeber) – so the value received from ADC (e.g. 3000) shall be converted into range from 0 to 100

Configuration:

the cf_Min_ADC_Value and cf_Min_Joy_Value

also

cf_Max_ADC_Value and cf_Max_Joy_Value

Shall be taken from configuration file

To map a value from one range to another, use linear scaling.

Given ranges

- ADC range:

  - cf_Min_ADC_Value = 0

  - cf_Max_ADC_Value = 4095

- Joystick (output) range:

  - cf_Min_Joy_Value = 0

  - cf_Max_Joy_Value = 100

General formula

Joy_Value =

((ADC_Value - cf_Min_ADC_Value) *

(cf_Max_Joy_Value - cf_Min_Joy_Value)) /

(cf_Max_ADC_Value - cf_Min_ADC_Value)

+ cf_Min_Joy_Value;

With your specific values

Joy_Value = (ADC_Value * 100) / 4095;

Notes (important in embedded systems)

Use integer or float carefully:

  - If using integers, you may want rounding:

  - Joy_Value = (ADC_Value * 100 + 2047) / 4095; // rounded

- Clamp the input to avoid overflow:

if (ADC_Value < cf_Min_ADC_Value) ADC_Value = cf_Min_ADC_Value;

if (ADC_Value > cf_Max_ADC_Value) ADC_Value = cf_Max_ADC_Value;

return value : This shall provide if the function was executed successfully (0) or not (1).