Architecture Document (STM32 Controller):

Author : Sanjyot Kadu

Date : 07-02-2026

Version No : Archi_doc_001

# Change History:

| Date | Version No | Change History |
| --- | --- | --- |
| 07-02-2026 | Archi_doc_001 | First Revision |

# Subject :

This document describes different layers of the architecture of the software to be prepared in modular approach.

Same Architecture can be used as a base for any Controls system used in STM32 or any other Family.

# Architecture:

Configuration:

appConf.c and appConf.h

ROBOTIC ARM

Application:

ROVER

Configuration

drivConf.c

drivConf.h

Driver Layer

MiddlewareConfiguration  (midConf.c and mindConf.h)

RC_Driver.C

RC_Driver.h

RC Driver

Middleware

CAN OPEN Protocol:

CAN_Open.c

CAN_Open.h

Joystick_Module:

JoyMod.c.

joyMod.h

DC_Motor_Control_Module:

DCMotCntrMod.c.

DCMotCntrMod.h

Stepper_Motor_Control_Module:

STPMotCntrMod.c.

STPMotCntrMod.h

Hardware Layer (STM CONTROLLER)

CAN.c and CAN.h

CAN Driver

analogVoltage.c

analogVoltage.h

Voltage IO Driver

PWM IO Driver

PWM.c  and PWM.h

Dig.c  and Dig.h

Digital  IO Driver

UART Driver

UART.c  and UART.h

ADC.c  and ADC.h

ADC Driver

## Hardware Layer:

This is actual HARDWARE, FOR SOFTWARE development this layer is used only for testing purpose (a programmer or test engineer can flash the code on the hardware and test if it’s working fine)

## Driver Layer:

This is the layer which will collect the data from hardware and then process it so that this is useful for Middleware to consume. For example, The ADC drivers shall read the ADC Values from HARDWARE and return it in case the function “getADCVal(channel No)” is called.

## Middleware:

This layer shall receive data from Driver layer and process it so that it is well defined for APPLICATION to use it.

For example, the Middleware can call the function from Driver layer to get the ADC Data and then take this data as input for the function and process in “JoyScaledVal joyADCtoScaledValConv(ADC Value)” and provide back the Joystick values.

## Application:

Application will use the Middleware, APIs to process to get the main application functionalities working for example the function

“runDcMotor(Dir, Val)” function use the function from middleware like getJoystick() and runDCmotor() to move the rover in respective direction with respective speed.

## Configuration

There will be 3 types of  configuration :

appConfiguration

middlwareConfiguration

DriverConfiguration

These will hold the dat with which the funtions of the respective lyer will take help .

For example, in Middleware configuration we can define the temperature- resistance table, so if we change the values inside configuration, it will affect the how the function executes.

Configuration gives us strength to change the behavior of the function without changing the function definition.